DATE: 08-OCT-24

RIG:       ADM-688

WELL: ZULF-1262

# CaCl2 Brine & SAMAA MUD WASH

# Objective:

      - Prepare Cacl2 brine, spacers & mud wash

      - Displacement from OBM to Brine

      - Spot SAMAA wash in the open hole/screen annulus

# Procedure:

- Finish mixing fresh Cacl2 brine in pit #4
        400 bbl. Drill water 
         40 Big Bag CaCl2.

- Transfer all OBM (10 bbl.) in pit #10 to pit #1

- Once start POOH with reaming BHA, start cleaning, slug #9, Slug #10

- Transfer 80 BBL 86 PCF Brine from Pit#2  to Slug #9 (Viscous Push Spacer)

- Transfer 80 BBL 86 PCF Brine from Pit#2  to Slug #10 (Non-Viscous Push Spacer)

- keep 320 BBL 86 PCF Brine in pit #4 (final volume will confirm by SAMAA engineer)

# Mix Spacers:

- Slug #9   Viscous Push Spacer: Mix the following with the same order:

| NO. | Chemical | Quantity | notes |
| --- | --- | --- | --- |
| 1 | Drill water | 7 bbl. | Final M. WT=84 PCF |
| 2 | HEC | 4 sacks | Mix Slowly |
| 3 | Slick-5 | 4 Drums |  |

- Slug #10    Non-Viscous Push Spacer: Mix the following with the same order:

| NO. | Chemical | Quantity | notes |
| --- | --- | --- | --- |
| 1 | Drill water | 7 bbl. | Final M. WT=84 PCF |
| 2 | Slick-5 | 4 Drums |  |

# Displace to Cacl2 brine pumping Sequence:

      - Keep sand trap pits Isolated (short system).

      - Empty & Clean trip tank from any OBM.

      - Fill up Trip tank with Brine

# Displacement Pumping sequence:

      - Pit #5		  20 bbl. Safra Oil / Low Rheology Mud

      - Slug #9	  80 bbl. of viscous Push Spacer

      - Slug #10	  80 bbl. of non-viscous Spacer

      - Pit #3 & Pit #4 Brine Active pit#3

          - Line up pit#3 as active pit for CaCl2 Brine and top up from pit #4 to pit #3.

      - OBM Return	 pit #7 then pit #5 then pit #6

      - Once viscous spacer returns to the surface, divert the flow overboard from shakers till a clean brine is observed.

      - Once a clean brine reaches the surface, cease pumping & monitoring closely for any well losses.

# Cleaning surface lines:

        - Flush Chock Manifold with brine.

        - Clean all surface lines.

        - Clean ditch line from shakers to mud pits. (dirty water to Pit Slug #9)

      - Dump and Clean Slug #10 from any spacers.

        - Transfer 80 bbl. Brine from pit#3 to slug #10

        - Mix 4 Sacks of HEC into slug #10

      - Prepare CaCl2 brine in pit #4:

        - Transfer 400 bbl. of 86 PCF CaCl2 brine from Brine tank to pit #4

        - Then add 35 bbl. of Drill water for final M. WT=84 PCF

# .

# SAMAA WASH:

# Mixing SAMAA wash pill as per SAMAA engineer:

- Pit #2    SAMAA engineer to confirm and finalize the volumes and mix the pill.

SAMAA Wash must be pumped downhole within 1-2 hours after mixing.

| NO. | Chemical | Quantity | notes |
| --- | --- | --- | --- |
| 1 | Brine | 320 bbl. | 86 PCF |
| 2 | Orca-B | 32 Drums |  |
| 3 | Orca-4 | 15 Drums |  |

# Spot in the open hole/screen annulus

# Pumping sequence:

      - Pit Slug #10	  60 bbl. HEC Pill

      - Pit #4		  10 bbl. Brine

      - Pit#2		  SAMAA Wash Pill.

      - Pit #4		  10 bbl. Brine

      - Pit Slug #10	  10 bbl. HEC Pill

      - Pit #4 & Pit #3  Active pit for brine suction and return.

          - Displace with brine to Spot SAMAA wash as per SAMAA engineer.

      - Flow Check for 30 Min Before POOH, DON`T POOH IF LOSSES MORE THAN 20 BPH.

# .

# Mud Pits:

| Pit NO. | Details | Notes |
| --- | --- | --- |
| Pit#1 | OBM |  |
| Pit#2 | SAMAA WASH |  |
| Pit#3 | CaCl2 brine |  |
| Pit#4 | CaCl2 brine |  |
| Pit#5 | Low Rheology OBM |  |
| Pit#6 | OBM |  |
| Pit#7 | OBM |  |
| Pit#8 | OBM |  |
| Pit#9 | Viscous Push Spacer / HEC Pill | HEC PILL after Displacement |
| Pit#10 | Non-viscous Spacer / HEC Pill |  |

# Safety Precautions:

- Follow all safety protocols during the mixing process.

- Ensure all personnel involved wear appropriate PPE (Personal Protective Equipment).

- In case of any issues or uncertainty, contact the mud engineer immediately.