DATE: 30-SEP-24

RIG:       ADM-688

WELL: Zulf-1262

# Displacement of Barite free OBM plan

Important Notes Before Displacement:

-Dump all sea water from Slug Pit #9.

-Safra Oil Addition: After pumping the last Hi-WT sweep of Barite OBM, add 50 bbl. of Safra Oil into Slug Pit #10.

-Sand Trap Pits: Keep the sand trap pits isolated until the displacement process is complete. Ensure the system is lined up in a short configuration.

-Trip Tank Management: Do not empty the trip tank from Barite OBM until Pit #8 is completely empty from Barite-Free OBM while Displacing.

Pumping Sequence Plan

- Spacer:

  - Pump 40 bbl. of Safra Oil from Slug Pit #10.

- Barite-Free Rheliant OBM:

  - Pit #8: Begin pumping Barite-Free Rheliant OBM from Pit #8.

  - Pit #6: Once Pit #8 is depleted, switch to Pit #6 to continue pumping Barite-Free Rheliant OBM.

Receive Barite OBM Returns:

- Start by receiving the return of Barite OBM into Pit #1.

- After Pit #1, direct the returns into Slug Pit #10, followed by Slug Pit #9.

- At this stage, Pit #8 will be empty of Barite-Free OBM. Begin transferring Barite OBM returns into Pit #8.

Transition to Barite-Free OBM:

- Once Safra Oil is observed on the surface:

  - Line up the sand trap pits in a long system.

  - Use Pit #6 as the active and return pit for the Barite-Free Rheliant OBM.

Mud Pits Management Plan

| Section | Details |  |
| --- | --- | --- |
| Pit#1 | Barite OBM (Active) | Barite OBM (return) 1 |
| Pit#2 | Empty for brine |  |
| Pit#3 | Empty for brine |  |
| Pit#4 | Barite-Free OBM (Hi-WT) |  |
| Pit#5 | Barite-Free OBM (Hi-WT) |  |
| Pit#6 | Barite-Free OBM Active #2 |  |
| Pit#7 | Barite-Free OBM Reserve |  |
| Pit#8 | Barite-Free OBM Active #1 | Barite OBM (return) 4 |
| Pit#9 | Empty | Barite OBM (return) 3 |
| Pit#10 | Safra oil | Barite OBM (return) 2 |

Thanks, Work safe

For any question call mud engineer