# L1 8-1/2” Reaming Run 
Mud Pits Plan

# Objective:

- Treat active system with shale inhibition and bridging material while reaming.

- Increase Mud active system volume to prepare Sealing pill (CSG go pill).

- Prepare Hi-Vis to sweep hole every 10 stands.

# Procedures:

- While RIH from surface with reaming

- Increase Mud active system volume to prepare Sealing pill (CSG go pill).

  - Transfer all barite free Rheliant OBM from Pit #7 to Pit #5

  - Transfer all barite Rheliant OBM from Pit #3 to Pit #7

  - Open Equalizer between pit #7 and pit #6

  - Line up active pits as pit #7 Return and pit #6 Suction.

- While Reaming down:

- Treat active system with shale inhibition and bridging material while reaming:

Mix the following in active pit #7

  - One time only:

    - Marble Medium  1 Big Bag

  - Hourly treatment:

    - Poroseal 	 4 drum

    - Baracarb 50 	 1 sack

    - Baracarb 25 	 1 sack

    - Steal Seal SF 	 1 sack

- Prepare Hi-Vis to sweep hole every 10 stands:

  - In Slug #10: prepare Hi-Vis as needed:

  - Transfer 70 bbl from active pit to slug #10, then mix:

    - VG Plus  	  1 sack

    - VG Supreme    1 sack

- At TD:

- Increase active system M-WT to  96 PCF by adding  10,000 lb Barite

- While POOH to window:

- prepare Sealing pill (CSG go pill).

  - Close Equalizer between pit #7 & pit #6:

  - Keep Pit #6 as active pit, Suction and return.

  - Maintain pit #7 volume 300 bbl.

  - Mix the following in pit #7 as CSG go pill :

    - Baracarb 50 		 30 sack

    - Baracarb 25 		 30 sack

    - Steal Seal SF 		 30 sack

    - Marble Medium 	 1 Big Bag

    - Lubraglide		 6 sacks

  - Transfer all barite free Rheliant OBM from Pit #7 to Pit #5

  - Transfer all barite Rheliant OBM from Pit #3 to Pit #7

  - Open Equalizer between pit #7 and pit #6

  - Line up active pits as pit #7 Return and pit #6 Suction.

------------------------------------------------------------------------------------------------------

# Procedures: while RIH with reaming :

Thanks and work SAFE