DATE : 28-SEP-24

RIG:       ADM-688

WELL: Zulf-1262

# Barite free OBM Management plan

1. Receive B.F.OBM from Boat (Rawabi-22)
   - Quantity: 2200 bbl
   - Storage: Distribute the B.F.OBM into pits 4, 5, 6, 7, and 8.

2. Check Mud Parameters and Weight (M.wt)

- Test and Treat mud to maintain mud parameters as per program.
   - Target Mud Weight: Adjust OBM in pits 4, 5, 6 & 8 to 83 PCF.

3. Weight Up Pit 7 for Hi-WT Pills
   - Target Mud Weight: Weight up the OBM in pit 7 to 98 PCF.

6. Fill Up Sand Trap
     - Fill the sand trap with 200 BBL of 83 PCF OBM from pit #8

- Keep Sand Trap ISOLATED After filling up. Ensure that all gates are sealed and direct the mud system line to the short system until the displacement to barite-free Rheliant OBM is complete.

# Mud Pits Management Plan

| Section | Details |  |
| --- | --- | --- |
| Pit#1 | Barite OBM |  |
| Pit#2 | Brine |  |
| Pit#3 | Brine |  |
| Pit#4 | B.F. OBM (till displacement) | then Sama wash pill |
| Pit#5 | B.F. OBM Reserve |  |
| Pit#6 | B.F. OBM Active |  |
| Pit#7 | B.F. OBM Hi-WT |  |
| Pit#8 | Barite OBM return from displacement |  |
| Pit#9 | Safra oil | then NON-vis push pill |
| Pit#10 | Slug | then vis push pill |

Note: Further game plans will be developed and provided as necessary based on operational requirements.